地物二  林士棋  93622014

Site Effect Evaluation of Surface Ground using Nakamura's

Technique and its applications

摘要

一般在探討場址效應時，通常會使用頻譜比法，又分為雙站頻譜比法 ( Borcherdt, 1970) 和單站頻譜比法 ( Nakamura, 1989 )。

雙站頻譜比法原理是為了消除震源和路徑效應，留下所要探討的場址效應，所以需要在測站附近找一個岩盤站，再加上能激發沖積層測站的地震不一定能激發岩盤站，所以岩盤站的紀錄不易獲得，即為此法困難之處。

Nakamura ( 1989 ) 提出單站頻譜比法經驗式，利用微地動資料之水平向與垂直向之傅氏頻譜比，來估算該站場址效應。而Lermo and Chavez-Garcia ( 1993 ) 從理論層狀構造來進行探討，認為此法也適用於地震之S波紀錄。Lachet and Bard ( 1994 ) 亦利用理論計算來證實此法之實用性。公式為

利用單站頻譜比法，可以得到主要的放大頻率及放大倍率，依據Nakamura ( 1996 ) 提出Kg值計算方法，可以快速簡便的分析場址的液化潛能指數。

公式為

Kg值愈大，則液化發生機率愈高；愈低，表示液化較不易發生。

Reference:

Nakamura, Y., （1989）. A method for dynamic characteristies estimation of subsurface using microtremor on the ground surface, QR of RTRI, 30, no.1, February,25-33.

Nakamura, Y.(1996), “Real Time Information Systems for Seismic Hazards Mitigation UrEDAS, HERAS and PIC” , Quarterly Report of Railway Technical Research Institute (RTRI), Vol.37,NO. 3,112-127.

Nakamura, Y., 2000. “Clear identification of fundamental idea of the Nakamura’s technique and its applications” ,12th World Conference of Earthquake Engineering, Auckland, New Zeeland, Proceedings, No. 84:8.