Empirical models for site- and region-Dependent

Ground-Motion Parameters in Taipei

Chun-Te Chen

Abstract

We analyze the site response of the Taipei basin using the records obtained by the Taiwan Strong Ground Motion Instrumentation Program network. The site response is obtained in terms of spectral ratios calculated by dividing of the site spectrum by the reference spectrum estimated for a hypothetical “very hard rock” site. The empirical source scaling and attenuation models for the Taiwan region are used for the reference spectra calculation. We calculated peak ground accelerations and response spectra for Taipei using stochastic simulation technique on the basis of the empirical models. The simple w-squared Brune’s point-models provides a satisfactory estimation of ground-motion parameters for rock site. Effects of local site response are considered by means of empirical soil/bedrock spectral ratios calculated as ratios between spectra of actual earthquake records and those modeled for hypothetical “hard rock” site.

Reference

Vladimir Yu. Sokolov, Chin-Hsiung Loh, M.EERI; and Kuo-Liang Wen , 2000. Empirical Study of Sediment-Filled Basin Response: The Case of Taipei City, Earthquake Spectra, Volume 16, Issue 3, pp. 681-707

Vladimir Yu. Sokolov, Chin-Hsiung Loh, M.EERI; and Kuo-Liang Wen , 2001. Empirical Models for Site- and Region-Dependent Ground-Motion Parameters in the Taipei Area: A Unified Approach, Earthquake Spectra , Volume 17, Issue 2, pp. 313-331